J40	FLEXIBLE SHEET TANKING / DAMP PROOFING

To be read with Preliminaries / General conditions.

TYPES OF TANKING / DAMP PROOFING

J40/290	HIGH DENSITY POLYETHYLENE / POLYPROPYLENE STUDDED DAMP PROOFING

- Substrate :

-	Preparation : In accordance with manufacturer’s recommendations.

- Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Cavity Drain R7

- Stud height : 7mm.

- Colour : Translucent

- Fixing : Vertical surfaces only, fixed through studs.

-	Fasteners : RIW Brick Plugs.

-	Fixing centres :1000mm maximum, staggered.

-	Sealing : RIW Sealing Rope around RIW Brick Plugs before insertion.

- Joints : Minimum overlap ; 70mm, flanged side of roll  or  200mm end / cut rolls.

-	Sealing : RIW Sealing Tape in lap.

- 	Drainage components : See Clauses 382 & 388. Control panels and alarms available.

- Accessories : See  product data sheet.

WORKMANSHIP

J40/310 	WORKMANSHIP GENERALLY

- Condition of substrate :

-    Clean and even textured, free from voids and sharp protrusions.

-    Moisture content : compatible with damp proofing / tanking.

- Air and surface temperature : Do not apply sheets if below minimum recommended by membrane manufacturer.

- Condition of membrane at completion :

-	Neat, smooth and fully supported, dressed well into abutments and around intrusions.

-	Completely impervious and continuous.

-	Undamaged. Prevent puncturing during following work.

- Permanent overlying construction : Cover membrane as soon as possible.

J40/320	INSPECTION

- Give notice : Before covering any part of membrane with overlying construction.

J40/360	JUNCTIONS WITH PROJECTING DPC’s / CAVITY TRAYS

- Adjoining surfaces : Clean and dry.

- DPC’s / Cavity trays : Lap and fully bond / seal with sheeting.

- Laps ( minimum ) : 150mm.

- Bonding / Sealing : RIW Overtape.

J40/370A	PIPES, DUCTS, CABLES, ETC :

- Where these pass through sheeting, make junctions completely impervious, following details as recommended for the purpose by the sheet manufacturer.

J40/382 	CAVITY DRAINAGE CHANNELS

- Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Aqua Channel