J40	FLEXIBLE SHEET TANKING / DAMP PROOFING

To be read with Preliminaries / General conditions.

TYPES OF TANKING / DAMP PROOFING

J40/290	HIGH DENSITY POLYETHYLENE / POLYPROPYLENE STUDDED DAMP PROOFING

- Substrate : …………………

-	Preparation : In accordance with manufacturer’s recommendations.

- 	Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Plasterdrain.

- Stud height : 5mm.

- Colour : Clear.

- Fixing : Fix between studs.

-	Fasteners : RIW Plaster Plugs.

-	Fixing centres : Minimum : 13 number per square metre, in a diamond pattern.

-	Sealing : RIW Sealing Rope around RIW Plaster Plugs before insertion.

- Joints : Minimum overlap ; 2 No studs. Provide row of fixings at overlap position.

-	Sealing : RIW Overtape

- 	Drainage components : See Clauses 382 & 388. Control panels and alarms available.

- Accessories : RIW Overtape ; see also product data sheet

RIW Cementseal Primer ; anti-lime treatment

WORKMANSHIP

J40/310 	WORKMANSHIP GENERALLY

- Condition of substrate :

-    Clean and even textured, free from voids and sharp protrusions.

-    Moisture content : Compatible with damp proofing / tanking.

- Air and surface temperature : Do not apply sheets if below minimum recommended by

- Condition of membrane at completion :

-	Neat, smooth and fully supported, dressed well into abutments and around intrusions.

-	Completely impervious and continuous.

-	Undamaged. Prevent puncturing during following work.

- Permanent overlying construction : Cover membrane as soon as possible.

J40/320	INSPECTION

- Give notice : Before covering any part of membrane with overlying construction.

J40/370A	PIPES, DUCTS, CABLES, ETC :

- Where these pass through sheeting, make junctions completely impervious, following details as recommended for the purpose by the sheet manufacturer.

J40/382 	CAVITY DRAINAGE CHANNELS

- Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Aqua Channel