F30	ACCESSORIES / SUNDRY ITEMS FOR BRICK / BLOCK / STONE WALLING

To be read with Preliminaries / General conditions.

FLEXIBLE DAMP PROOF COURSES / CAVITY TRAYS

F30/330	DAMP PROOF COURSE – POLYMERIC.

- Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Sheetseal 9000 DPC.

INSTALLATION OF DPC’s / CAVITY TRAYS

F30/415	HORIZONTAL DPC’s

- Placement : In continuous lengths on full even bed of fresh mortar, with 100mm laps at joints and full laps at angles.

- Width : At least full width of leaf unless otherwise specified.

Edges of DPC not covered with mortar or projecting into cavity.

- Overlying construction : Immediately cover with full even bed of mortar to receive next masonry course.

- Overall finished joint thickness : As close to normal as practicable.

F30/425	GROUND LEVEL DPC’s

- Joint with damp proof membrane : Continuous and effectively sealed.

F30/435	STEPPED DPC’s IN EXTERNAL WALLS

- External walls on sloping ground : Install DPC’s not less than 150mm above adjoining finished ground level.

465	SEALING DPC’s …………………

- Overlaps and junctions : Seal with RIW Jointing Tape.