J40	FLEXIBLE SHEET TANKING / DAMP PROOFING

To be read with Preliminaries / General conditions.

TYPES OF TANKING / DAMP PROOFING

J40/140  	LOOSE LAID POLYETHYLENE GAS RETARDANT DAMP PROOFING

- Substrate : …………………

- Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Gas Seal Black.

- Thickness / Gauge : 0.6mm

- Joints :

- Surfaces to be joined : Clean and dry beyond full width of joint.

- Laps ( minimum ) :150mm.

- Sealing : RIW Gas Seal Tape placed centrally in the lap, with RIW Lap Tape laid over the edge of the upper sheet by 50mm, and adhered onto the lower sheet.

- Accessories : See product data sheet.

WORKMANSHIP

J40/310	WORKMANSHIP GENERALLY

- Condition of substrate :

-	Clean and even textured, free from voids and sharp protrusions.

-	Moisture content : Compatible with damp proofing / tanking.

- Air and surface temperature : Do not apply sheets if below minimum recommended by membrane manufacturer.

- Condition of membrane at completion :

-	Neat, smooth and fully supported, dressed well into abutments and around intrusions.

-	Completely impervious and continuous.

-	Undamaged. Prevent puncturing during following work.

- Permanent overlying construction : Cover membrane as soon as possible.

J40/320	INSPECTION

- Give notice : Before covering any part of membrane with overlying construction.

- Independent validation and integrity testing required.

J40/360	JUNCTIONS WITH PROJECTING DPC’S / CAVITY TRAYS

- Adjoining surfaces : Clean and dry.

- DPC’s / cavity trays : Lap and fully bond / seal with sheeting.

-	Laps ( minimum ) : 150mm

-	Bonding / Sealing : RIW Gas Seal Tape and RIW Lap Tape.

J40/370	PREFORMED COLLARS FOR PIPES, DUCTS, CABLES. ETC.

- Manufacturer :	As membrane manufacturer.

- Product reference : RIW Preformed Pipe Collars / RIW Gas Cloaks.

-	Sealing : RIW Gas Seal Tape and RIW Lap Tape.

- 	Completed junctions : Impervious.