J40	FLEXIBLE SHEET TANKING / DAMP PROOFING

To be read with Preliminaries / General conditions.

TYPES  OF  TANKING / DAMP  PROOFING

145J40/2862	LOOSE LAID WELDABLE POLYEHTYLENE GAS RETARDANT DAMP PROOFING

- Substrate : ……………..

-	Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

- Product reference : RIW Gas Seal HC

- Thickness / Gauge : 0.5mm

- Colour: Black

- Joints :

-	Surfaces to be joined : Clean and dry beyond full width of joint.

- 	Laps (minimum) : 100mm

-	Jointing : Hot-air welded.

-	Sealing : RIW Lap Tape. Roll to provide gas tight joints.

WORKMANSHIP

J40/310	WORKMANSHIP GENERALLY

- Condition of substrate :

-	Clean and even textured, free from voids and sharp protrusions.

-	Moisture content : Compatible with damp proofing / tanking.

- Air and surface temperature : Do not apply sheets if below minimum recommended by sheet manufacturer.

- Condition of membrane at completion :

-	Neat, smooth and fully supported, dressed well into abutments and around intrusions.

-	Completely impervious and continuous.

-	Undamaged. Prevent puncturing during following work.

- Permanent overlying construction : Cover membrane as soon as possible.

J40/320	INSPECTION

- Give notice : Before covering any part of membrane with overlying construction.