F30	ACCESSORIES / SUNDRY ITEMS FOR BRICK / BLOCK / STONE WALLING

To be read with Preliminaries / General conditions.

FLEXIBLE DAMP PROOF COURSES / CAVITY TRAYS

F30/360	GAS RESISTANT DPC’s / CAVITY TRAYS

-	Manufacturer : RIW Ltd, 487/488 Ipswich Road, Slough, Berkshire SL1 4EQ

Tel : +44 1753 944200

E-mail : technical@riw.co.uk

Web : www.riw.co.uk

-	Product reference : RIW Gas Seal GR DPC.

INSTALLATION OF DPC’s / CAVITY TRAYS

F30/415	HORIZONTAL DPC’s

- Placement : In continuous lengths on full even bed of fresh mortar, with 100mm laps at joints and full laps at angles.

- Width : At least full width of leaf unless otherwise specified.

Edges of DPC not covered with mortar or projecting into cavity.

- Overlying construction : Immediately cover with full even bed of mortar to receive next masonry course.

- Overall finished joint thickness : As close to normal as practicable.

F30/425	GROUND LEVEL DPC’s

- Joint with damp proof membrane : Continuous and effectively sealed.

- Use RIW Gas Seal Tape in lap, with RIW Lap Tape over end of upper sheet.

or

- Weld to DPM.

F30/435	STEPPED DPC’s IN EXTERNAL WALLS

- External walls on sloping ground : Install DPC’s not less than 150mm above adjoining finished ground level.

465	SEALING DPC’s …………………

- Overlaps and junctions : Seal as for Clause 425.

495	GAS RESISTANT DPC’S / CAVITY TRAYS

- 	Joint treatment : Use unjointed wherever possible, otherwise lap at least 150mm and seal to form a gas and watertight installation.

-	Joint with damp proof membrane : Overlap DPC / cavity not less than 150mm.