J40	APPLIED TANKING / DAMP PROOFING

To be read with Preliminaries / General conditions.

EXECUTION  continued.

J30/361	JUNCTIONS WITH STRUCTURESEAL

- RIW Sheetseal 226 to be overlapped by 150mm onto the applied RIW Structureseal.

- Seal together using 50 x 5mm bead of RIW Sealing Compound, in centre of lap.

- Apply remaining RIW Sheetseal 226, as required, in accordance with J40 /190

.